7-2 Project Two: Network Design Rationale

Blake Merry

Southern New Hampshire University

CYB-210 Programming Language

Prof. Jamilah Telon

April 19, 2026

## IP Networking

The network was redesigned to make it more efficient and easier to manage while keeping proper separation between different parts of the environment. The original Subnet1 and Subnet2 were combined into a single network, NetworkA (192.168.1.0/28). Instead of maintaining multiple small subnets, combining them reduces complexity and makes addressing easier to manage.

At the same time, Network 3 (192.168.1.16/28), which represents the newly acquired company, was kept separate. This separation is important because it prevents devices from freely interacting across networks and helps reduce risk if one network becomes compromised.

A dedicated server network (192.168.1.32/29) was also added to isolate infrastructure services. Even though there is only a single server in this arrangement, separating it from user devices makes the design cleaner and more secure. It ensures that services like DHCP are not mixed in with general client traffic, which can make troubleshooting and management easier.

Routing between networks was handled using static routes. Each router was configured with routes to reach the networks on the other side of the point-to-point link. This allows devices in NetworkA, Network3, and the server network to communicate with each other. Static routing works well here because the network is small and does not require the overhead of dynamic routing protocols.

This structure also reduces broadcast traffic and improves control over how data flows between networks, which makes the overall network more stable and easier to manage.

## Subnet Masks

Subnet masks were chosen based on how many devices each network needed to support. Both NetworkA and Network3 use a /28 subnet mask, which allows up to 14 usable IP addresses. This is enough for the number of PCs and printers in each network without leaving many unused addresses.

The server network uses a /29 subnet mask, which provides fewer addresses but is appropriate since only a small number of devices exist in that network segment. The router-to-router connection uses a /30 subnet mask, which is ideal for point-to-point connections because it only requires two usable addresses.

This approach avoids over-allocating address space while still leaving room for limited growth, which helps keep the network efficient without wasting IP addresses.

## DHCP Infrastructure

DHCP was centralized on Server_Main to simplify address management. Instead of configuring each device manually, client systems can automatically receive an IP address, subnet mask, and default gateway.

Because DHCP requests don’t cross routers by default, the “ip helper-address” feature was configured on the router interfaces for NetworkA and Network3. This allows DHCP requests from both networks to be forwarded to the server, ensuring that devices in each subnet still receive addresses even though the DHCP service is located on a different network.

This approach improves scalability and consistency, since all address assignments are managed from a single location instead of being configured individually on each device.

## Network and Port Address Translation Protocols

Network Address Translation (NAT) was not required for this internal network design since all devices operate within private address spaces and communication is handled through routing. However, NAT would typically be implemented at a network boundary to allow internal devices to access external networks while preserving private IP addressing.

This approach improves security by preventing direct exposure of internal systems and allows multiple devices to share a single public IP address, which is a common consideration in larger or internet-connected environments.

## Device Address Strategy

A mix of DHCP and static addressing was used depending on the device type. PCs were configured to use DHCP, which makes the network easier to manage as devices are added or removed. Printers were configured with static IP addresses so they remain consistent and can always be reached at the same address. This approach matches common industry practice, where end-user devices are dynamic but shared resources are fixed.

## Conclusion

Overall, the network design focuses on keeping things simple while still following best practices. Networks are segmented where it makes sense, addressing is efficient, and services are centralized to reduce management overhead. At the same time, routing and DHCP are configured in a way that allows everything to communicate reliably without adding unnecessary complexity.